Case 2: Error in Transmission

Data = 100100, Generator Polynomial (Key) = x3 + x2 + 1 (1101)

Receiver Side
Let there be an error and code word received at the receiver side 100000001.

#include <iostream>

#include <string>

#include <cmath>

#include <algorithm>

using namespace std;

#define int long long int

// Function to convert integer to binary string

string toBin(int num) {

// Handle case when number is 0

if (num == 0) return "0";

string bin = "";

while (num) {

// Append '1' or '0' based on least significant bit

bin += (num & 1) ? '1' : '0';

// Shift right to process next bit

num = num >> 1;

}

// Reverse string since bits were added in reverse order

reverse(bin.begin(), bin.end());

return bin;

}

// Function to convert binary string to decimal integer

int toDec(string bin) {

int n = bin.size();

// Handle empty string

if (n == 0) return 0;

int num = 0;

for (int i = 0; i < n; i++) {

if (bin[i] == '1') {

// Compute power of 2 for each '1' in binary string

num += 1 << (n - i - 1);

}

}

return num;

}

// Function to compute CRC and print remainder and codeword

void CRC(string data, string key) {

int n = key.length();

if (n == 0) {

cout << "Error: Key cannot be empty" << endl;

return;

}

// Convert binary strings to decimal integers

// Generator polynomial (key)

int gen = toDec(key);

// Original data

int code = toDec(data);

// Append (n - 1) zeros to the data to make space for CRC bits

int dividend = code << (n - 1);

// Calculate the position to start XOR (most significant bit position)

int shft;

while ((shft = (int)log2(dividend) - n + 1) >= 0) {

// Extract top 'n' bits of dividend, XOR with generator polynomial

int rem = (dividend >> shft) ^ gen;

// Replace top bits in dividend with XOR result (remainder)

dividend = (dividend & ((1 << shft) - 1)) | (rem << shft);

}

// Final codeword is the original data with the remainder appended

int codeword = (code << (n - 1)) | dividend;

// Print results

cout << "Remainder: " << toBin(dividend) << endl;

cout << "Codeword : " << toBin(codeword) << endl;

}

signed main() {

string data = "100100";

string key  = "1101";

CRC(data, key);

return 0;

}

Output

Remainder: 1

Codeword : 100100001